Faculty of Engineering, Computing and Mathematics

School of Computer Science and Software Engineering

CITS4419: Mobile and Wireless Computing

DSR:

A Raspberry Pi Implementation

Author:	Ash Tyndall	Asra Alsahib	Bo Chuen Chung

Dayang Abang Mordian	Hui Li Leow	Max Ward

Raphael Byrne	Timothy Raphael	Vincent Sun

Zhiqiang (Cody) Qiu

Supervisor:	Prof. Amitava Datta

Semester 2, 2013

- Table of Contents

1.	The project	4

2.	DSR Architecture	4

2.1.	Components	4

2.2.	Optimizations	4

3.	Documentation	4

3.1.	DSR Packet	4

3.1.1.	Attributes	5

3.1.2.	Packet representation	5

3.2.	DSR Routing	5

3.2.1.	Constants and Attributes	5

3.2.2.	Packet construction	7

3.2.3.	Network methods	7

3.2.4.	Routing methods	7

3.2.5.	Acknowledgement	9

3.2.6.	Send Packet	9

3.2.7.	Receive packet	9

3.2.8.	DSR Updates	9

3.3.	Route Cache	10

3.3.1.	Constant and Attributes	10

3.3.2.	Routing	11

4.	Simulation	11

4.1.	Network Simulation	12

4.2.	Real-time simulation	12

5.	Results	12

6.	Discussions	12

Appendix: DSR Test Cases	13

# The project

The goal of the project is to implement a Dynamic Source Routing (DSR) protocol. The DSR implementation (described in Section2) is written using Python 3.3.2. The hardware and software used in this project is tabulated in the following table.

| Hardware/Software | System Requirements |
| --- | --- |
| Raspberry Pi | Linux platform: Raspbian, Debian Memory: 512MB RAM CPU & GPU Power: Rechargeable battery LAN: Wi-Pi |
| Cnet simulator | Windows Vista, 7 and above Linux and UNIX platforms Macintosh OS-X 10.4 and above |

Table 1: Hardware and Software requirement

# DSR Architecture

This section will cover the components

## Components

## Optimizations

# Documentation

The DSR algorithm is implemented in python. This section will cover the documentation of the python implementation for the project.

## DSR Packet

Messages sent to each DSR node are embedded in the form of packet. The types of messages are summarized in Table 1. This section will describe the packet class (dsr_packet.py).

| Message Type | ID | Descriptions |
| --- | --- | --- |
| REQUEST | 1 | Route request to initial the routing |
| REPLY | 2 | Route reply for the route request received |
| ERROR* | 3 | Route error |
| SEND | 4 | Send Message |
| ACK | 5 | Acknowledgement for the message received |
| ERREQ* | 6 | Error route request to propagate a broken link |

Table 2: Message type (* denotes message type not in used)

### Attributes

| Attributes | Descriptions |
| --- | --- |
| Type | Packet’s message type represented by its corresponding ID as shown in Table 2. |
| Path | The path of the packet from source node to destination node. |
| Contents | The contents of the packet. If message type is send and ack, the content will be the actual message; for other message types, the content will be toID. |
| ID | The identifier of the packet. By default, this is set to be -1. |
| FromID | The identifier of the DSR source node. By default, this is set to be -1. |
| OriginatorID | The ID of the original packet. By default, this is set to be -1. |
| OriginatorNodeID | The ID of the node that first create the original packet. By default, this is set to be -1. |
| toID | The identifier of the DSR destination node. By default, this is set to be -1. |
| brokenLink | A tuple (A, B) that represents the broken link between the path A and B. By default, this is set to be (-1, -1). |

Table 3: DSR packet struture

Note: ID, OriginatorID and will always be a positive integer. When a node broadcast a packet, it resets the packet’s FromID and toID to -1.

### Packet representation

The packet is translated as string, passed from a DSR node to the network layer, which is represented in the following format:

Type| Path | Content| ID| fromID| originatorID| toID| brokenLink | originatorNodeID

where each item in Path and brokenLink are separated by ‘>’.

When a DSR node received the packet string from the network, the string is parsed into the packet object using the method from_str(packetStr).

## DSR Routing

### Constants and Attributes

| Constants | Descriptions |
| --- | --- |
| MAX_transmissions | The maximum number of transmissions taken before sending route error. |
| MAX_time_between_ack | The maximum time of waiting for an acknowledgement before retransmitting. |
| MAX_time_between_request | The maximum time DSR node should wait for a route reply before broadcasting the route request again. |

Table 4: Constants

| Parameters | Descriptions |
| --- | --- |
| Node_addr | The address of the DSR node. |

Table 5

| Attributes | Descriptions |
| --- | --- |
| ID | The identifier of this DSR node. This ID is set to be node_addr. |
| Next_packet_id | The id of next packet, initialized to be 0. |
| Received_queue | Keeps track of the packets which are ready to be received by this DSR node on a first in first out basis. |
| Send_queue | Keeps track of packets which are ready to be sent by this DSR node on a first in first out basis. Each item in the send_queue is represented in the form (contents, toID). |
| Send_buffer | Buffer that keeps track of route request packet, waiting for the route reply. Each item in the buffer is represented in the form (msg, broadcast_msg, start, counter). Note: - msg is the packet to send - broadcast_msg is the broadacasted request for msg - start is the starting time for which the packet is sent - counter is the number of times the packet has been sent. |
| Done_buffer | Buffer that keeps track of packets that are sent to this DSR node. |
| Outbox | A list of packets represented as string, which are ready to be sent on the network. Each item in the outbox is represented as tuple in the form (str(pkt), toId). |
| Awaiting_acknowledgement_buffer | Buffer that stores a list of sent packets that are waiting for acknowledgement. Each item in the buffer is represented as (pkt, start, timetransmitted), Note: - start is the starting time for which the packet pkt is being processed - timetransmitted is the number of times the packet has been transmitted. |
| Route_cache | The route cache that store the path for sending message (refer to Section 3.3. Route Cache). |
| Seen_errors | A set of tuples in the form of (fromID, originatorID), where the error of the originator packet sent by fromID has already been seen by this DSR node. |
| Seen_route_request | A set of route request which have been already been processed by this node. Each item is represented as a tuple (fromID, originatorID). |
| Already_received_msgs | A set of messages sent which have already been processed by this node. These messages are intended for this node to receive. Each item is represented as a tuple (origintatorID, originatorNodeID). |

Table 6: Node Structure

### Packet construction

Make_packet(type, path, contents): The method construct a packet by taking in packet type, path and contents as parameters. A packet’s id and originatorID are the current value of next_packet_id. Every time the method is called, next_packet_id will increment by 1 and return the newly made packet.

Make_packet_o(type, path, contents, originator, originatorNode): This method is used to construct a new packet that acknowledge the originators. Similar to make_packet, except that the packet’s originatorID and originatorNodeID are the respective parameters originator and originatorNode.

### Network methods

Network_broadcast(pkt): This method broadcasts pkt to its neighbour nodes. Pkt.fromID and pkt.toId are reset to -1. This str(pkt) is appended to the outbox queue.

Network_sendto(pkt, toID): This method is called when the DSR node is sending pkt to toID. Firstly, the pkt.fromID and pkt.toID are updated to this.ID and toID respectively. Then the str(pkt) is appended to the outbox.

### Routing methods

Route_request(msg): This method is used to process the route request (msg). Firstly, any msg.brokenLink will be removed from the route_cache. It will check two attributes of msg for route request: msg.contents and msg.path. The msg.content is the address of the recipient node. There are three cases in method:

- If this is equal to the DSR node’s ID, this node will make a route reply packet make_packet_o(REPLY, rev_path, toID, msg.originatorID, msg.originatorNodeID), where rev_path is the reversed  path and toID is the first address in the path. This packet is sent to the next address on the rev_path.

- However, if condition 1 is not met, and if this.ID is one of the addresses in msg.path, or the msg is contain in seen_route_requests, then the DSR node will do nothing to avoid any cyclic requests or seen route requests.

- If the first two conditions are not met, then the msg will be added to node’s seen_route_request. The DSR node will include itself in msg.path and broadcast the request using make_packet_o(REQUEST, msg.path, msg.contents, msg.originatorID, msg.originatorNodeID), together with the msg.brokenLink.

Route_reply(msg): This method is used to process the route reply (msg). There are two cases based on the msg.contents, which is used to identify the recipient’s node.

- If msg.contents is this.ID, then the DSR node will retrieve the contents from the send_buffer via remove_from_send_buffer(msg.originatorID). If the contents is not empty, this node use the contents to make a packet using make_packet(SEND, rev_path, contents), where rev_path is the reversed path of msg.path. This packet is sent to the next address in the rev_path, and also added to acknowledgement buffer via add_to_ack_buffer method.

- If condition 1 is not met, then this node will make a reply packet using make_packet_o(REPLY, msg.path, msg.contents, msg.originatorID, msg.originatorNodeID) and forward it to the next address in the path after itself.

Route _send(msg): This method is used to process the route send  (msg), which denote the sent packet that contains the intended message for the destination node. Firstly, the DSR node will send an acknowledgement to the sender of that msg. There are two cases here:

- If the destination of msg is this.ID, then this node will add the msg to the already_received_msgs and done_buffer if msg is not in already_received_msgs.

- If condition 1 is not met, then this node will packet a new packet using make_packet(SEND, msg.path, msg.contents). This packet is sent to the address of the next node in the path, and also added to acknowledgement buffer via add_to_ack_buffer method.

Route_discover(data, toID): This method is used by the node to discover route path. Firstly, this node will look at its route cache for path to toID via get_shortest_path(toID) – (Refer to Section 3.3 Route Cache). There are two cases here:

- If path to toID is in the route cache, then this node will make a send packet via make_packet(SEND, path, msg). This packet is sent to the next address in the path and also added to acknowledgement buffer via add_to_ack_buffer method.

- If no such path is found, this node will make a request packet pkt via make_packet(REQUEST, new_path, toID), where new_path is a list containing only this.ID. This node will broadcast the request packet and record the time of the broadcast, and append (data, pkt, time, 1) to send_buffer.

Route_discover_with_error(msg, originalPkt, brokenLink): This method is used to fix the broken link between two nodes in msg.path. Firstly, brokenLink will be removed from route_cache. This node will then make a new route request using make_packet_o(REQUEST, new_path, originalPkt.toID, originalPkt.originatorID, originalPkt.originatorNodeID), where new_path is a list that contains only this.ID. The request will also piggyback the brokenLink and broadcasted by this node so that the neighbouring nodes will remove the brokenLink from their route_cache. This node will record the time of sending the requestion, which is appended to send_buffer in the form (originalPkt.contents, request, start, 1).

### Acknowledgement

Msg_acknowledgement(msg): This method is used to process the acknowledgement (msg) in that this node is waiting for. In the acknowledgement msg, msg.contents contains id of the packet that is waiting to be acknowledged. Once the item in the awaiting_acknowledgement_buffer matches that id, it is removed from the buffer.

### Send Packet

Send_message(contents, toID): This method is used to add the to-be-send packet to the send_queue. (contents, toID) is appended to the send_queue.

### Receive packet

Receive_packet(pkt):This method is used to process the pkt sent to this node transported by the network. It will parse the string packet pkt which is then added to the receive_queue if and only if this node is the recipient of pkt. Additionally, this node will look at pkt.fromID and pkt.path to make sure that pkt is not from itself and add the pkt.path into the route_cache (via offer_route(pkt.path) – See Section 3.3 Route Cache) if the length of pkt.path is greater than 1.

Pop_inbox(): This method will pop and return all the received messages in the done_buffer that are directed to the DSR node.

Pop _outbox():This method will pop and return all the sent messages in the outbox that are sent from the DSR node.

Remove_from_send_buffer(ID):This method remove the packet from send_buffer based on the packet’s ID. It will loop through all the sent messages in the send_buffer to find the message with originatorID matching the ID. Once this message is found, the method will return the message and remove it from send_buffer.

### DSR Updates

Check_ack_buffer(): This method will update all items in awaiting_acknowledgement_buffer for route maintenance. It will loop through the acknowledgements ack in the awaiting_acknowledgement_buffer. There are two cases here:

- If the number of packet transmission to next node has exceeded the MAX_transmissions, this DSR node will broadcast the error message to its neighbours about the brokenLink, where brokenLink is (this.ID, next). This node will call route_discover_with_error(ack[pkt], brokenLink) in an attempt to fix the routing, and remove the ack from awaiting_acknowledgement_buffer.

- If the ack did not exceed the MAX_transmissions duration, but the time taken to receive the ack exceed the MAX_time_between_ack interval, then this node will send the packet again.

Add_to_ack_buffer(pkt): This method will add acknowledgement packet pkt to the waiting list. There are two cases for this method.

- If pkt is already in the awaiting_acknowledgement_buffer in the form (pkt, start, timetransmitted), it will be removed from the buffer. To retransmit the pkt, (pkt, new_start, timetransmitted) is added to the buffer, where new_start is the time of the retransmission.

- If condition 1 is not met, then (pkt, start, 1) is added to awaiting_acknowledgement_buffer, where start is the time of the transmission.

Check_send_buffer(): This method will update all items in the send_buffer. It will loop through the sent item (msg, originatorID, start, counter) in the send_buffer to check whether it exceeds the sent duration, MAX_time_between_request x counter. If this condition is met by an item, it will be removed from the send_buffer.  A new item (msg, originatorID, new_start, counter+1) is then appended to the buffer, where new_start is the new time of packet retransmission.

Update(): This method will update the attributes of the DSR node. When the method is called, this node will perform three actions as follows:

- Firstly, process items in ack_buffer and send_buffer via check_ack_buffer and check_send_buffer respectively.

- Next, process and pop all items in receive_queue. For each message msg in the receive_queue, the DSR node will ignore any messages that are from itself. For each message, there are five possible scenarios for each message type:

  - REQUEST: run route_request(msg)

  - REPLY: run route_reply(msg)

  - SEND: run route_send(msg)

  - ACK: run msg_acknowledgement(msg)

- Finally, process and pop the all items in send_queue. For each send message [in the form of (contents, toID) in the send_queue, the DSR node will call the route_discover(contents, toID).

## Route Cache

The route cache is represented using graph.

### Constant and Attributes

| Constant | Descriptions |
| --- | --- |
| MAX_DELTA | The maximum age of a link measured in milliseconds. |

Table 7

| Parameters | Descriptions |
| --- | --- |
| myID | The ID of the node that uses this route cache. |

Table 8

| Attributes | Descriptions |
| --- | --- |
| Edge_list | A collection of the edge list for graph representation. Each item in edge_list is represented in the form {A: {B1, B2, …}}, where the set {B1, B2, …} does not contain any duplicates, and node A is linked to B1, B2, and so on in the graph. |
| Edge_age | A collection of the age for each the edges in the graph. Each item in edge_age is represented in the form {A: {B: n}}, where the link from A to B has age n. |
| me | The identifier of the root node. This is set to be myID. |

Table 9

### Routing

Offer_route(route): The route from node A to B is represented as list in the form [A, …, B] with length n. This method will add the route information into the cache via call add_link(route[i], route[i+1]), where 0 ≤ i < n.

Add_link(fromID, toID): This method adds a single link (fromID, toID) to the route cache by adding it into the edge_list and also edge_age with the current time measured in milliseconds.

Remove_link(fromID, toID): This method remove a single link (fromID, toID) from the route cache by removing it from the edge_list and edge_age.

Get_shortest_path(toID): This method is used to find the shortest path to the destination node toID. It is used to expire any old links so that the route cache. This is done by removing the link that has an age greater than MAX_DELTA. To make sure route cache is up-to-date, this method will expire old links via expire_link(). Breath-first-search is used to find the single source shortest path to toID.

# Simulation

The simulator is used to simulate the communication between the nodes in DSR.  In this project, we have implemented a network simulator to run repeatable experiments for evaluating the performance of the implemented DSR. For real-time simulation, we have implemented the DSR in Raspberry Pi, a small mobile system.

## Network Simulation

The network simulator is used to simulate the communication between 5 mobile nodes. The communication matrix between nodes is initialized prior to the start of simulation. We currently initialized three communication matrixes in three distinct time stamp to represent which node can communicate to each other at different time stamp. There are 50 simulation steps take place in one simulation cycle. When the simulation starts, the first step is to randomly pick a source and destination nodes (the nodes are represented by integer 0-4), and, generate a random message in the form of 6 ASCII characters. Then, the network simulator passes these values to DSR to process the communication between the nodes. If the current iteration step matches the next network change, the network simulator will switch to the next communication matrix. Note that, the main purpose of the network simulator is only to test DSR functionalities in a fixed network topology environment.

## Real-time simulation

We perform real-time simulation using Raspberry Pi device as the mobile nodes. The purpose of this simulation is to test DSR functionalities in a dynamic network topology changes. This simulation mimics the network simulator, however, rather than using a fixed number of mobile nodes and a fixed communication matrix between the nodes, real-time simulation is adapting to the dynamic changes (i.e. the number of mobile nodes participate in the network change and the communication links between certain nodes may broken at any time)

# Results

DSR performance result is documented in the form of test cases. Our test cases cover the general aspects of DSR. These include:

- Route Discovery

- Route Request

- Route Cache

- Route Reply

- Route Maintenance

- Others

# Discussions

# Appendix: DSR Test Cases

Figure 1: Topology

*Note: The table below is based on the sample network topology above.

|  | Description | Expected Results | Actual Results |  |
| --- | --- | --- | --- | --- |
|  |  |  | Network Simulation | Real-time Simulation |
| Route Discovery | Automatic route discovery | DSR successfully performs automatic route discovery. | Successful The next simulation step showing that the source node automatically performs route discovery to destination node. | Successful Automatic route discovery executes every time the nodes are within each other’s communication range. |
|  | On-Demand route discovery | DSR successfully performs route discovery only on demand. No periodic broadcasting involved. | Successful The next simulation step showing that the source node automatically performs route discovery to destination node and no other periodic broadcasting identified. | Successful Automatic route discovery executes every time the nodes are within each other’s communication range and no other periodic broadcasting identified. |
| Route Request | Sending route request | DSR successfully allows nodes firstly search their own route cache to see whether they have stored route to destination node. If nodes have the route, then nodes successfully sends that route to source node, else, nodes broadcast the route request message to their neighbors and attach their own ID to the route request message. | Successful The upcoming simulation steps showing that the source node is using route cache information to determine the path to destination node. | Successful The mobile nodes showing that the source node is using route cache information to determine the path to destination node. |
|  | Route request frequency | DSR successfully allows source node to wait for a fixed amount of time before initiating another route request to avoid flooding the network. | Successful Waiting time for route request is identified. | Successful Waiting time for route request is identified. |
|  | Hop limit | DSR successfully controls the route request to neighborhood area to avoid unnecessary route reply message. | Successful The simulator defined a default communication matrix to determine which node can talk to each other at each time instance. This avoids unnecessary route reply message. | Successful The route request executes based on node discovery. DSR does not send route request if the neighborhood nodes are outside the communication range |
|  | Restricted Propagation of Route Request | DSR successfully increases hop count proportionately when receiving no route reply messages to avoid network congestion an unnecessary route reply messages (i.e. start with distance 2, then when no reply message, increase distance to 3 and so on). | Unsuccessful The communication between mobile nodes is determined by the communication matrices. The route request executes only when the communication matrix is true. | Unsuccessful The communication between mobile nodes is based on the communication range. If the nodes fall into each others’ communication range, route request executes. |
| Route Cache | Maintaining route cache information | Each node successfully maintains a route cache and remember the routes that it has learnt about | Successful The next simulation step which has similar source and destination nodes with previous simulation step showing that the source node is able to search its own route cache to destination node. | Successful The same mobile nodes which try to communicate with each other again showing that the source node is able to search its own route cache to destination node. |
|  | Route cache updates | DSR successfully maintains up-to-date information in the route cache for each node. | Successful The next simulation step which has similar source and destination nodes with previous simulation step showing that the source node is able to use up-to-date route cache information. | Successful The same mobile nodes which try to communicate with each other again showing that the source node is able to use up-to-date route cache information . |
|  | Route cache has no available information | DSR successfully initiates a new route discovery when no route cache information available. | Successful The first simulation step showing that the source node initiate route discovery when it is unable to obtain any information from its own route cache. | Successful The first communication between mobile nodes showing that the source node initiate route discovery when it is unable to obtain any information from its own route cache. |
|  | Caching overhead routing information | DSR successfully allows node P to store overhear routing information in its route cache from node B to node C. DSR is also successfully use this information when node P receive route request from node B to node C. | Successful The next simulation steps are able to use the route to destination node that is identified by the other nodes in the previous simulation step. | Successful The mobile nodes are able to use the route to destination node that is identified by the other nodes in the previous communication. |
| Route Reply | Complete route discovery | DSR successfully sends route reply message from destination node that has route to destination in its route cache. | Successful Identified. | Successful Identified. |
|  | Partial route discovery | DSR successfully sends route reply message from intermediate node that has route to destination in its route cache. | Successful Identified. | Successful Identified. |
|  | Waiting before reply | All nodes successfully wait for a random amount of time and listen to the traffic before sending route reply message to avoid network congestion and packet collisions. | Successful Waiting time before route reply is identified. | Successful Waiting time before route reply is identified. |
|  | Accumulated route reply | DSR successfully sends route reply message back to the source node using accumulated route (the nodes through which it has passed). | Successful Accumulated route reply is identified in the packet details. | Successful Accumulated route reply is identified in the packet details. |
|  | Route reply storm | DSR successfully avoids route reply storm (many nodes try to send route reply for the same destination which may flood the network). | Successful Waiting time before route reply is identified. | Successful Waiting time before route reply is identified. |
| Route Maintenance | Alternative route for broken route | DSR successfully uses alternative route stored in route cache when priority route is broken. | Successful Identified. | Successful Identified. |
|  | New route discovery for broken route | DSR successfully discovers new route when priority route is broken and route cache has no alternative route stored. | Successful When the route is broken and route cache is empty, the next simulation steps showing that the source node no longer has priority route stored in its route cache and it starts a new route discovery | Successful When the route is broken and route cache is empty, the mobile nodes showing that the source node no longer has priority route stored in its route cache and it starts a new route discovery |
|  | Active acknowledgment | DSR successfully retransmits packet for a fixed number of times if no acknowledgement received. | Successful The simulation output showing that DSR keep transmitting | Successful Re-transmitting is identified on the output when the source node failed to send message to destination node |
|  | Passive acknowledgement | DSR successfully sends acknowledgement to node A when node A overhear the forwarding of the packet to node B and knows that node B successfully received the packet. | Successful Identified. | Successful Identified. |
|  | Spreading route Error Message | DSR successfully sends error message if the nodes do not receive any acknowledge-ment after retransmit message for a fixed number of times. | Successful Identified. | Successful Identified. |
|  | On-Demand route maintenance | DSR successfully operates route maintenance only on demand and no periodic broadcasting involved. | Successful Retransmitting packets occurs only when the source node failed to reach destination node. No periodic broadcasting identified. | Successful Retransmitting packets occurs only when the source node failed to reach destination node. No periodic broadcasting identified. |
|  | Changing in communication pattern | Number of overhead packets increases and DSR is successfully performs new route discovery and new route discovery packets are the overhead packets. | Successful Identified. | Successful Identified. |
|  | Packet salvaging | DSR successfully indicates to the other nodes that the packet sent has been salvaged when the node receives route error message and re-send the packet that cause route error. | Successful Packets are attached with broken link information to let the other nodes know that the packet has been salvaged. | Successful Packets are attached with broken link information to let the other nodes know that the packet has been salvaged. |
|  | Automatic route shortening | DSR successfully performs automatic route shortening. Destination node informs source node that it can ignore several intermediate nodes. | Successful Identified. | Successful Identified. |
| Others | Drop packets when nodes are static | DSR successfully drops the number of overhead packets to zero when the nodes are static and all routes have been discovered. | Successful Identified. | Successful Identified. |
|  | Energy-efficient | DSR successfully shows more energy-efficient and does not congest the network with too many control messages. | Unsuccessful Unable to determine the energy efficiency because the simulation output is in the text format. | Unsuccessful Unable to determine the energy efficiency of mobile nodes at the moment. Longer and depth testing are needed to determine this attribute. |
|  | Unique ID | DSR successfully assigns unique ID for each node. | Successful Route reply messages contain unique ID of the source and destination node | Successful Route reply messages contain unique ID of the source and destination node |
|  | Promiscuous mode of operation | DSR successfully allows each node to overhear or not to overhear other nodes' transmission. | Successful The next simulation steps are able to use the route to destination node that is identified by the other nodes in the previous simulation step. | Successful The mobile nodes are able to use the route to destination node that is identified by the other nodes in the previous communication. |